.po 0

#PROGRAMMING THE HV-2000 FOR FUN... AND PROFIT!

----------------------------------------------#

In the January issue of Remark, I introduced Heath's new  HV-2000

PC  Voice Card.  As an inexpensive kit that goes together  easily

in  an  evening  and  has many  novel  and  useful  applications,

Marketing  expected  it to be a popular product,  but  it's  been

selling at over twice their forecast!

I  suspect  that many HUGgies have found or are  creating  unique

applications for it that hadn't even occurred to us.  At the  end

of  this article, you'll find information on a contest that  will

give  you  a  chance to win a prize in return  for  sharing  your

applications  with  others.  At the conclusion  of  the  contest,

HUG  will compile a library of programs and routines and make  it

available on disk.

In the meantime, Heath's Technical Correspondence Department  has

received a number of requests for more information on how the HV-

2000  can be made to sing and produce sound effects.  So, in  the

next  few sections I'm going to offer three short BASIC  programs

to  get  you  started,  as well as  some  programming  hints  and

information on interfacing directly with the hardware,  bypassing

the device driver.  This discussion assumes that you have already

read the HV-2000 manual.

.pa

MAKING THE HV-2000 SING

It apparently wasn't clear in the manual that simply specifying a

Note  attribute  (nN)  does not cause any sound  to  be  created.

Instead,  it  determines the pitch of any  phonemes  subsequently

spoken.   For example, to sing 'DO, RE, MI' from the DOS  prompt,

you could use the command

SPEAK "={N d o w pa 2N r a ie pa 4N m e ie pa}"

NOTE:   The  '=' in the string returns all  attributes  to  their

default values.  This includes defaulting to Mode 0 (transitioned

inflection), which is not appropriate for singing.  However, when

the device driver sees the first note attribute in the string, it

automatically  switches to Mode 1 (instantaneous inflection),  so

that the notes will occur at the proper pitch.

If  you are writing a program to sing a song, it is usually  best

to  use phonetic spellings of the words (in braces), because  you

then  have control over the timing.  You can change the  duration

of  individual phonemes in a word and/or add additional  phonemes

until the duration of each word is just right.

As  an example, Program 1 sings part of Lara's Theme, then  plays

the tune with two-note 'chords' (actually alternating notes).

To  aid in picking note values, Figure 1 is an illustration of  a

section of a piano keyboard, with corresponding N values for four

octaves.

.pa

PRODUCING SOUND EFFECTS

Many  sound effects consist of producing some basic  sound,  then

altering  one or more attributes.  For example, to  simulate  the

ringing  of  a  bell, you could output a fricative  to  make  the

striking  sound, followed by some voiced phoneme for  the  'ring'

sound,  output  at  successively lower amplitudes  to  cause  the

ringing to decay.  For example

SPEAK "={FA d a CA a AA a 8A a 6A a 5A a 4A a 3A a 2A a 1A a A}"

Note  the  attempt  to simulate an  exponential  decay.   To  get

fancier, you might try altering not only the amplitude with time,

but the filter and/or inflection frequency slightly as well.  Try

improving  on  the  above example and see  how  closely  you  can

simulate a bell.

Another  example is Program 2, which simulates a scene from  Star

Wars.  Here Filter frequency, Amplitude, Rate, and Inflection are

the attributes being altered.

A small problem exists if you want to produce a continuous  sound

effect, such as the steady hiss of a gas leak.  The device driver

was  written with voice synthesis as its primary  purpose.   When

speaking  a string of text, you don't want the last phoneme  that

was  output  to  continue sounding - you want  it  to  terminate.

Otherwise,  a string ending with the word 'hello',  for  example,

would  sound a continuous 'O'.  A string ending in  a  fricative,

such  as 'test', would terminate, but the final 't' would not  be

output until the next voiced phoneme or Pause phoneme was output.

Therefore,  the  device  driver  automatically  outputs  a  Pause

phoneme  when it encounters the carriage return at the end  of  a

string.

You  could  output  a  long string of  'h'  phonemes  to  give  a

reasonably long hissing sound, as with the BASIC line

OPEN "O",1,"HV":PRINT#1,"{h h h h h h h h h h h h h h h}":CLOSE#1

But  even  if you loop this line over and over, there will  be  a

short  pause  inserted  on each iteration.  So  if  you  want  to

concentrate on sound effects, and find this to be a problem, here

is  a patch you can make to the device driver.  Be sure  to  copy

the original VOICE.SYS to a backup file, say VOICE.BAK, first.

.pa

To make the patch, type the following at the DOS prompt:

DEBUG VOICE.SYS <return>

FCS:2210 L3 90 <return>

W <return>

Q <return>

Then  copy  the  patched VOICE.SYS to your  root  directory,  and

reboot.  Then type

SPEAK "hello"

You've  got a continuous sound now, don't you?!  To shut  it  up,

type

SPEAK "{pa}"

As you can see, using the patched device driver requires that you

make  a  point  to terminate your strings with  a  Pause  phoneme

yourself.  But now if you want that hissing sound, just type

SPEAK "{h}"

Program  3, which requires that you make the above patch to  work

properly, will produce a wind storm simulation that is convincing

enough to send a shiver down your spine.  It simply generates the

wind  sound  with  an h phoneme, and randomly  ramps  the  filter

setting  up and down to simulate changes in wind speed.  To  make

the  effect  more realistic, the amplitude is made to  track  the

'speed'.  (From BASIC, you can stop the sound by writing a  Pause

phoneme  directly  to the hardware using the  BASIC  command  OUT

&H300,0.   This assumes that your board is jumpered  for  address

300H.)

After making the patch, run Program 1 again, and notice that  the

pauses  between  the two-note chords have disappeared,  giving  a

more pleasing sound.

Program  2  could be simplified now, by  outputting  the  desired

phoneme  once,  and  using a FOR-NEXT loop to  step  through  the

descending  Filter frequencies.  This would also make it easy  to

randomly select the phoneme used to produce the basic sound.

.pa

BYPASSING THE DEVICE DRIVER

Although  the device driver and software utilities included  with

the  HV-2000  were written to provide a  simple,  yet  versatile,

interface to the product, there may be applications where  you'll

want to bypass the device driver, and interface with the hardware

directly.  In doing so, you should be aware that the SSI-263  has

a  number of idiosyncrasies that were discovered and  compensated

for  during  the writing of the device driver.  So  if  something

doesn't  work  quite as expected, you may need  to  experiment  a

little.   If you're still game, the following information  should

be helpful.

Refer  to  Table  1  for  the  function  of  each  of  the  eight

addressable  registers.  Most of the bits shown in the table  are

self-explanatory, except for the following:

DR1,DR0  define  the phoneme duration, where 0,0 is  the  #longest #

duration.

P5-P0  specify the desired phoneme per Table 2.

The C bit, when set, disables (powers down) the SSI-263 chip.  It

also  selects  the inflection mode in the following  manner.   If

bits  DR1=DR0=1  when  the  C  bit  is  taken  low,  Mode  0   is

established.   If  DR1=1  and DR0=0 when C goes low,  Mode  1  is

selected.  (The other two combinations should not be used.)

T2-T0 define the articulation speed.

Reading  from any of the eight registers returns, in bit  7,  the

Ready  status of the chip.  In polled operation, on outputting  a

phoneme to the Duration/Phoneme register, bit 7 will read low for

the  duration  of  the phoneme.  When it returns  high,  you  can

output the next phoneme.

If  you don't load the device driver, be sure to perform a  Write

to  register  6  to enable the board at  the  beginning  of  your

program.

Keep in mind that the SSI-263 was designed for speech  synthesis,

not sound effects.  The sound effects that can be simulated using

the  various  sounds  and attributes that are  available  can  be

useful in some situations, but are definitely limited.

.pa

THE CONTEST

Because  there are so many potential applications, it's  hard  to

set  rules or restrictions for program entries to  this  contest.

Your  entry  could be a game, an educational program, such  as  a

typing  teacher or language tutor, an aid to the  handicapped,  a

talking appointment calendar, or any other useful,  entertaining,

and/or  novel application.  It need not be long or  complex.   It

may be written in Assembly, BASIC, or some other common language,

and  can take advantage of the features of the device driver,  or

talk  to  the hardware directly.  The HUG staff and  I  will,  as

judges,  just have to look over (and listen to) all the  entries,

and fight it out amongst ourselves!

The  deadline for submittals will be May 31, 1988.  You need  not

be  a  member of HUG to enter, and you may submit more  than  one

entry.  All entries should be submitted on disk, and will  become

the property of Heath Users' Group.  The following prizes will be

awarded:

First place:   $300 HUG Buck

Second place:  $200 HUG Buck

Third place:   $100 HUG Buck

The winners will be announced, and their entries presented, in  a

subsequent issue of REMARK.

So get started, and good luck!##############