Note:  To obtain a printed copy of this file, put  your  printer

on-line, log onto this disk, and type PRINTME.

(1)

Heath HD-1780 Controller Program

Features

* Transient or memory-resident (pop-up) operation

* HV-2000 Voice Card support

* User-selectable color combinations

* On-screen date and time

* Ability to see most internal settings

* Display of all ten named bearings

Introduction

This  program  allows you to take full advantage  of  the  serial

interface  in  your HD-1780 IntelliRotor.

Most  functions are activated by use of the  computer's  function

keys  and  keypad  keys, and menus appear at the  bottom  of  the

screen to define the action of the function keys.  In addition, a

PROMPTS  window  provides useful information on the use  of  many

functions, and displays error messages.

You  can  scroll  through  the countries  database  and  see  all

pertinent information conveniently displayed instantly.

The  program may be run as a transient or installed to be  memory

resident, allowing it to be popped up over a logging program, for

example.

(2)

Running the program

Type HD <enter> to run the program.

There  are three 'switches' that may be specified in the  command

line  following  the  HD command.  If more  than  one  switch  is

specified, they may be entered in any order, but each switch must

be preceded by a slash (/).  Type HD/ <enter> to get  information

on switch usage.

The program defaults to the use of serial port COM1 unless the /2

switch  is  used, and to the use of color unless  the  /M  (Mono)

switch is used.  If you run this program on a laptop with an  LCD

screen, you will probably need to specify the /M switch.

The third switch is /R (Resident).  Using this switch causes  the

program to install itself as a memory resident (pop-up)  utility.

To  invoke the program, you must hold both SHIFT keys down  while

pressing the H key.  The program will only pop up if the  display

is in an 80 column text mode (mode 2, 3 or 7).  The first time it

is  popped  up  it will take a second to paint  itself  onto  the

screen;  subsequent  pop-ups will be  instantaneous.   A  Restore

function  (described later) is automatically performed each  time

the program is popped up.

When  the program is first run or installed, it looks for a  file

named  HDLIST.DAT in the current directory.  This  file  contains

the country database in a particular format.  If the file is  not

found,  the  program  will read the data from  the  HD-1780,  and

create  the  file.   Once created, the program will  be  able  to

simply read the file into memory.

Notes:

1)  If  you make any changes to the country list in  the  HD-1780

using the EDIT function, you should delete HDLIST.DAT so that

the program will create a new file that includes the  changes

you've made.

2)  Since  the program looks for HDLIST.DAT in the directory  you

are logged on to when you run or install the program,  always

make sure that you are in the proper directory at that  time,

so that the program won't create extra copies of the file  in

other directories.  To help prevent this possibility, if  the

program  doesn't find the file in the current  directory,  it

will  ask you if it should create it there.  Responding  with

other than the Y key will cause the program to be exited.

3)  If  installing  the program as a pop-up, pop it up  once  (by

holding both Shift keys down while pressing H).  It reads  or

creates  the  HDLIST.DAT file at this point.   Press  ESC  to

return to DOS.

(3)

General Operation

If  something happens to cause an error message to be  displayed,

the  console bell will sound, and you will be prompted  to  press

any key to confirm that you are aware of the error.

In  particular,  the program checks for  communication  with  the

controller  at various times.  If it does not get a response,  it

will  tell you so, and ask that you correct the  situation,  then

press any key.  (If you press ESC, the program will be exited.)

Most  functions described below allow you to use the ESC  key  to

change your mind if you make a mistake.

Information displays

The  screen displays the title block and local clock/calendar  on

the  top  line.  Note that the time and date are  read  from  the

computer, not the controller.

The  second area from the top of the screen contains  information

relating to the target country or bearing.  These items are self-

explanatory, but two things need to be explained.

1)  The  distances shown are those read from the controller  when

the disk file HDLIST.DAT was created, and are in the unit  of

distance selected at that time.  Therefore, if you select the

other  unit of measure, you will need to  delete  HDLIST.DAT,

and  let the program re-create it.  (If you have the  program

installed  as a pop-up, you must also reboot  and  re-install

it.)

2)  The  target time will only be correct if your HD-1780 is  set

to display the same time as your computer.  If you selected 0

at the Hours from UTC? prompt in the Setup menu, so your  HD-

1780  would read out in UTC time, than your computer must  be

set for UTC also.

The  next section from the top contains information  on  internal

settings  and  current  status.  Most of  these  are  also  self-

explanatory.   The  window labeled # Pls@ ACC PW relates  to  the

'auto-rotate  parameters' described in the HD-1780  manual.   The

first  number  is  the selected setting (the  shifted  digit  key

pressed  to make the selection).  This is followed by the  number

of degrees from target at which the rotor motor is pulsed to slow

it down.  The next number is the attempted accuracy, and the last

number is the pulse width used to slow the motor.

The  local  bearing  (BRG)  will go blank  when  the  rotator  is

turning,  since  the bearing cannot be read from  the  controller

during  this  time.   As soon as rotation  stops,  this  will  be

updated.

(4)

Automatic selection

When you make a selection by one of the following means, the path

will  default to SHORT.  To begin rotation to the short  bearing,

press  ENTER.  If you want the long path heading, press  the  TAB

key first.  It toggles between short and long path bearings,  and

the  distance  (if  displayed) and  bearing  are  updated.   When

rotating,  pressing  the SPACE bar will stop  rotation;  pressing

ENTER again will resume.

The  area  near  the center of the screen displays  up  to  seven

countries and their prefixes.  You can use the up- and down-arrow

keys  to  scroll  through the list.  As you  do  so,  the  target

information is updated.

Pressing ENTER will cause the controller to begin rotating to the

currently highlighted country.  The PgUp and PgDn keys scroll  by

seven entries at a time, and the Home and End keys go to the  top

and bottom of the list, respectively.

To select a named bearing, press F7.  You will be asked to  press

a  digit key to select one of the ten named bearings  shown  near

the  bottom  of the screen.  If a country was highlighed  in  the

list,  it will be unselected, and the only information  displayed

in the Target area will be the bearing you entered.  Press  ENTER

again to begin rotation.

To  select a particular bearing in degrees, press F8.  Enter  the

bearing,  and  press ENTER.  If a country was highlighed  in  the

list,  it will be unselected, and the only information  displayed

in the Target area will be the bearing you entered.  Press  ENTER

again to begin rotation.

To  select by country name, press F9.  Enter the country name  or

the  first few letters of it, and press ENTER.  The  first  match

will  be  positioned  to the top of the  list,  and  highlighted.

Pressing ENTER again starts rotation.

To  select by prefix, press F10.  As in the HD-1780, an  explicit

search  is  made first, and if it fails, and implicit  search  is

made.  So you may enter a prefix, partial prefix, or entire  call

sign.   The  first  country  that  matches  your  entry  will  be

displayed with its prefix list.  If it is the one you want, press

ENTER.   If  not, press SPACE to search for the next  match.   To

begin rotation, press ENTER.

(5)

Manual rotation

The  left- and right-arrow keys may be used for manual  rotation.

(If  your  keyboard has an auto-repeat feature, you may  want  to

turn the keyboard click off if the repeated clicks are annoying.)

NOTE: There  is a possibility of a problem using  the  left-arrow

key  IF your IntelliRotor is configured for  North-centered

operation,  AND the rotor is at its CW limit.  This is  due

to  the  way  the left- and right-arrow  keys  operate  the

controller.   They  do  so by sending  a  'go  to  bearing'

command,  a 180 <CR> to turn counter-clockwise, and  a  179

<CR>  to turn clockwise.  If the rotor is at its CW  limit,

it  should  read 179 degrees, but due to  tolerances,  even

right  after  calibration,  may read 180 or  181.   If  so,

sending a command to rotate to 180 will cause it to respond

with  a beep to say "I'm already there!".  It  is  unlikely

that  this will occur often, but if it does, press the  CCW

manual  rotate  button on the HD-1780 briefly,  then  press

REMOTE.

Entering new named bearings

While  the  ten  named bearings in the  controller  may  only  be

changed  using the controller's STORE key, you may change any  of

the ten named bearings shown on the screen.  Press F3, enter  the

number of the memory you wish to change, enter the new name,  and

press ENTER.  You will be asked to enter a new bearing, or  press

ENTER  to use the current bearing (in the BRG window).  (You  may

also press ESC to leave the bearing unchanged.

NOTE: The new named bearings will be lost if you exit the program

if  it  is  being run as a transient.  If  installed  as  a

memory-resident  'pop-up', they will be lost when  you turn

your  computer off or reboot, or if you use the F4  Restore

key.

Restoring information

Pressing  F4 causes the program to read information from the  HD-

1780  concerning  its current settings, and update  the  display.

This information includes the named bearings.

(6)

Color selection

If  you have a color monitor, you will appreciate the ability  to

customize  the colors to your liking.  Temporary changes  may  be

made during an operating session to relieve eyestrain, or changes

may be made permanent.

Press  F6  and a new menu will appear, defining sections  of  the

screen,  and  you will be prompted to make  a  selection.   After

doing so, the selection begins blinking in the menu line, and you

are  prompted  to use the arrow keys to select  colors.   The  Up

Arrow  and  Down Arrow keys change background colors,  while  the

Left  Arrow and Right Arrow keys change foreground colors.

Note:  The  program  automatically prevents any  selections  that

would  cause any part of the screen to  become  invisible.

For  example,  the  program won't allow the  choice  of  a

foreground   (background)   color  of  brown   until   the

background  (foreground)  color is  changed  to  something

other than brown.

After  changing  a  section,  press  ESC.   The  selection  stops

blinking, and you can choose another section to change, or  press

ESC again to leave the color selection mode.

If you hold either SHIFT key down while pressing ESC to return to

the Ready! prompt, the changes will be made permanent. *

Entering a custom Title

F5  is labeled Title.  Press F5, and enter a new title.   If  you

press ESC, the old title will return; pressing ENTER accepts  the

new  one.  If you want to make this title permanent, hold  either

SHIFT  key down while pressing ENTER. *  (Note that  the  current

color selections will also be saved by this action.)

* Important:   The   program  will  be  saved  in   the   current

directory, under the name FM.COM.

Exiting the program

Press the ESC key to leave the program.  If it was being run as a

transient  program, you will be asked to confirm this  action  by

pressing 'Y'.

(7)

HV-2000 Voice Card Support

The  controller  program supports the Heath HV-2000  voice  card,

allowing use of the HD-1780 by blind operators.  Refer to the HV-

2000 manual for installation of the VOICE.SYS device driver,  and

note that, for the the controller program to speak properly,  the

WORDS.EXC  file included on this disk must be copied to the  root

directory,  so that it will be loaded at boot-up with the  device

driver.   (If  you have already created your own  WORDS.EXC  file

with  the  NEWORDS utility, make sure to rename it  first  so  it

won't be written over.)

The voice may be turned on and off by the use of the F1 key.  The

voice  will say 'Voice on' or 'Voice off'.  When on, pressing  F2

(Speak) will cause a voice menu to appear, and you will be  asked

what  information  you  want  spoken.   Press  the  corresponding

function  key.   After  the information  is  spoken,  the  normal

function menu will return.

When  the  voice is on, all prompts that appear  in  the  Prompts

window  will be spoken, with the exception of those  relating  to

color selection.  In addition, some other actions will be spoken.

The voice parameters are initialized when the program is  invoked

(transient mode) or installed (resident mode).  When in  resident

mode, but not popped up, you can change the voice parameters from

the  DOS prompt by using the SPEAK command furnished on  the  HV-

2000 Utility Disk.

Miscellaneous Notes

If you have only a single serial port, and need to share it  with

a  printer or other device using an A-B switchbox, note that  the

program fully restores the selected serial port upon exiting.

If you experience problems with this program, either in transient

or  pop-up  modes,  it  may  be  due  to  other,  memory-resident

programs.   Uninstall them and try installing them one at a  time

to determine which one is causing the problem.