Control2 - Pre-release

Control2 implements 2 loops of up to PID control using the IBM DACA board.

The controller is structured as described in the UnkelScope manual, with the

addition of an adjustable anti-windup control and the ability to use a specified

set point as the command signal rather than an analog input value.  In this

initial version, you can not store the data or the 'setup' configuration.  We

will release the full version of this program with the ability to save a

data file that can be read and plotted by UnkelScope after the control is done.

You can order this as "Control2 for IBM DACA" at a cost of $49.

Control2 is structured like the configuration program.  The main menu allows you

to select the operation.  In this release, you cannot save data or save the

setup configuration.

Control2 Inputs.  Control2 will read the "ibmcan.aux" file written by the

UnkelScope configuration file to determine the DACA settings.  If this file

is not present, some default settings will be assumed.

Controller Setup Parameters.  For each controller you select the controller type

(Open Loop or PID) and then set the appropriate parameters.  See your

UnkelScope Users Guide for a description of most of the parameters.  The Windup

limit is used to keep the integral portion of the controller from destroying

performance.  This is a very simple limit and is applied to the entire signal

NOT just to the integral component.

Sampling Parameters.  You select the speed, the triggering option and the

mode of sampling in this block.  THERE IS NO CHECKING OF WHETHER A SPEED YOU

SELECT IS MET BY THE PROGRAM.  For an IBM PC/XT at 4.77MHz, a speed of 200Hz

worked faithfully.  For an IBM PC/AT at 8MHz you may be able to reach speeds of

500Hz.  One way to check the speed is to select the mode where 1024 samples are

taken then the controller stops - you can then time how long the 1024 samples

take and see if it is correct.  Unless you are very close to the limit, if you

select too fast a rate, the actual speed will be 1/2 what you selected, so it

will be easy to detect the failure.

Program Output.  At speeds less then 0.5Hz (2 seconds between updating values)

the program will print the A/D input and D/A output values to the lower right

of the screen.  At higher speeds, there is no program output except for the

D/A values themselves.