; Copyright 1987  Unkel Software, Inc.

; callable library to handle IBM DACA

;

;  This library should work with any MicroSoft language that

;    uses the arguement passing strategy of MicroSoft Fortran 3.XX,4.XX

;    As required by MicroSoft C 4.XX, these routines save the contents of

;    the si and di registers.  If using C be careful to note that C counts

;    arrays from 0 where as Fortran counts arrays starting at 1.

;

;  In all cases the arguements passed are Integer*2 or arrays of Integer*2

;  When passing arrays, it is the programmers responsibility to

;   insure that there is enough space for the number of values stored.

;

;  It is also the programmers responsibility to make a call to SETUCA with

;   a proper starting address before accessing the board.  It is also

;   the programmers responsibility to send only valid values of the variables

;   as these routines do no checking to see if the parameters are within

;   bounds.

;

;  setuca(iadsta)            setup board at address iadsta

;                              you must call this before making any other calls

;                            if set as adapter 0, the correct base address is

;                              738 (=2E2 Hex)

;  pcadsc(iadval())          return channel 0-3 into array iadval(1,2,3,4)

;  pcdasc(idaval())          send array idaval(1,2) to d/a channels 0,1

;                            the integers in idaval must be properly scaled

;                            and prepared before calling the program

;  pcadda(iadval(),idaval()) sample a/d and send d/a - pcadsc,pcdasc

;  pcatod(istat,ivalue)      get a/d at istat(=256*channel #) immediately

;  pcdtoa(istat,ivalue)      output value ivalue to d/a (istat=256*channel #)

;  pcdgou(ivalue)            put value to digital output

;  pcdgin(ivalue)            get digital value

;  watica(nerr)              wait for counter overflow of counter 1

;                              if you are using this for timing and set the

;                              rate too fast, you will miss 'tics' there

;                              is no way of detecting this

;                            nerr is an error code.  It will be set to 0 unless

;                              there has been a keyboard abort.  A keyboard

;                              abort is generated by pressing the ESC key.

;  rdtica(ivalue)            return value in counter 1

;  sttica(idatt1,idatt2)     set up pacer clock

;                             the delay between overflows of counter 1 will be

;                             t=0.9775 E-6 * idatt1 * idatt2

;                            To use waitca, the values of idatt1 and idatt2

;                             should be greater than 15.

;  clint                     clear the interrupts

;  rdint(itint,icint,irint)  return the value of the interrupt flags

;  seint(ivalue)             set the interrupt register - if you set the

;                             interrupts active you must provide the interrupt

;                             appropriate interrupt service routine